Simulating Turtlebot3 in Gazebo using ROS 2

(https://emanual.robotis.com/docs/en/platform/turtlebot3/manipulation/)

TurtleBot3 Manipulation = TurtleBot3 + OpenMANIPULATOR + MoveIt

The TurtleBot3 Simulation Package requires the turtlebot3 and turtlebot3_msgs packages. Without these prerequisite packages, the TurtleBot3 Manipulator cannot be launched. Turtlebot3 manipulation means instead of only moving around, the robot can now drive, reach, pick, place, interact with objects.

Install Dependent ROS 2 Packages

- Open the terminal with Ctrl+Alt+T on the Remote PC.

- Install Gazebo
[Remote PC]

$ sudo apt install ros-humble-gazebo-*

- Install Cartographer
[Remote PC]

$ sudo apt install ros-humble-cartographer
$ sudo apt install ros-humble-cartographer-ros

- Install Navigation2
[Remote PC]

$ sudo apt install ros-humble-navigation2
$ sudo apt install ros-humble-nav2-bringup

Install TurtleBot3 Packages

Install the required TurtleBot3 Packages.

[Remote PC]

$ source /opt/ros/humble/setup.bash
$ mkdir -p ~/turtlebot3_ws/src
$ cd ~/turtlebot3_ws/src/
$ git clone -b humble https://github.com/ROBOTIS-GIT/DynamixelSDK.git
$ git clone -b humble https://github.com/ROBOTIS-GIT/turtlebot3_msgs.git
$ git clone -b humble https://github.com/ROBOTIS-GIT/turtlebot3.git
$ sudo apt install python3-colcon-common-extensions
$ cd ~/turtlebot3_ws
$ colcon build --symlink-install
$ echo 'source ~/turtlebot3_ws/install/setup.bash' >> ~/.bashrc
$ source ~/.bashrc

Environment Configuration

- Setup your ROS environment for the Remote PC.
[Remote PC]

$ echo 'export ROS_DOMAIN_ID=30 #TURTLEBOT3' >> ~/.bashrc
$ echo 'source /usr/share/gazebo/setup.sh' >> ~/.bashrc
$ echo 'source /opt/ros/humble/setup.bash' >> ~/.bashrc
$ source ~/.bashrc

- Install the packages for TurtleBot3 Manipulation.
[TurtleBot3 SBC]

$ sudo apt install ros-humble-hardware-interface ros-humble-xacro ros-humble-ros2-control ros-humble-ros2-controllers ros-humble-gripper-controllers
$ cd ~/turtlebot3_ws/src/
$ git clone -b humble https://github.com/ROBOTIS-GIT/turtlebot3_manipulation.git
$ cd ~/turtlebot3_ws && colcon build

$ sudo apt install ros-humble-dynamixel-sdk ros-humble-ros2-control ros-humble-ros2-controllers ros-humble-gripper-controllers ros-humble-moveit*
$ cd ~/turtlebot3_ws

colcon build

Install Simulation Package

Install the packages for TurtleBot3 Manipulation Gazebo simulation.

[Remote PC]

$ cd ~/turtlebot3_ws/src/
$ git clone -b humble https://github.com/ROBOTIS-GIT/turtlebot3_simulations.git
$ cd ~/turtlebot3_ws && colcon build

#How to run gazebo

$ ros2 launch turtlebot3_manipulation_gazebo gazebo.launch.py

To control the TurtleBot3 in the Gazebo simulation, the servo server node of MoveIt must be launched first.

$ ros2 launch turtlebot3_manipulation_moveit_config servo.launch.py

Launch the keyboard teleoperation node.

$ ros2 run turtlebot3_manipulation_teleop turtlebot3_manipulation_teleop

TIP

Following keys are used to control the TurtleBot3.

Use o|k|l|; keys to move turtlebot base and use 'space' key to stop the base
Use s|x|z|c|a|d|f|v keys to Cartesian jog
Use 1|2|3|4|q|w|e|r keys to joint jog.
'ESC' to quit.

Integrate Gazebo simulation with Rviz2